! for this guide to work you must have a working docker-open5gs program, cloned form the github repository of borijs131. See relevant documentation for the correct installation and setup of that program!

This is very simple:

- You must enter the webui container inside the following configuration file:

  - docker-open5gs/compose-files/roaming/docker-compose.yaml

- the way you do that is you copy paste the txt I will provide, right before the ‘networks’ field. Take care of the indentation, press space twice before copying the webui txt.

- Then you must do the following in your home directory of each vm:

  - nano /etc/hosts

  - to do this correctly follow the guide on this link: https://open5gs.org/open5gs/docs/tutorial/05-roaming/

  - I will provide the correct configuration inside the images folder

- After that you run the following command in both vms:

  - docker compose -f compose-files/roaming/docker-compose.yaml --env-file=.env up -d

- Then you enter the webui in your computer browser at:

  - http:<your_vm’s_ip>:9999

- there you add a subscriber, the configuration of the subscriber is located at the images folder

- then you must restart the db container:

  - docker restart db

- after that you should be seeing correct logs, to quickly check if the connection is successful, all you need to do is check the v-amf logs in the V-PLMN and the v-sepp and h-sepp logs in the V-PLMN and the H-PLMN respectively.

- If it is not working properly you probably did the subscription in the wrong vm/plmn. Try repeating the process with the webui and the ue subscription in the other vm, that should fix it.

- The program closes with:

  - docker compose -f compose-files/roaming/docker-compose.yaml --env-file=.env down

  - systemctl stop docker

  - systemctl stop docker.socket